| Feature | Microcontroller (e.g., Arduino, ESP32) | Single-Board Computer (e.g., Raspberry Pi) |
| --- | --- | --- |
| Architecture | A single chip containing a CPU, memory, and programmable I/O peripherals. | A complete computer built on a single circuit board, including RAM, CPU, and I/O. |
| Operating System | Typically runs "bare-metal" (no OS) or a lightweight Real-Time Operating System (RTOS). | Runs a full operating system (Linux, Windows IoT). |
| Processing Power | Low. Designed for simple, repetitive, and dedicated tasks. | High. Capable of handling complex logic, multitasking, and heavy data processing. |
| Power Consumption | Extremely low. Often designed for deep-sleep modes to conserve energy. Can run for weeks or months on battery. | High. Usually requires a continuous wired power supply to handle the CPU and OS. |
| Boot Time | Almost instantaneous (milliseconds). | Slower (20–45+ seconds) as it must boot a full OS. |
| Cost | Very inexpensive (often just a few dollars). | More expensive due to complex components and memory. |

### Why use a Microcontroller over a Single-Board Computer?

- Real-Time, Deterministic Control: Because there is no heavy operating system managing background processes or network updates, an MCU can execute code with exact, microsecond-level timing precision. This is critical for reading sensors or controlling motors where timing cannot be interrupted. Code can be recompiled across different CPU as long as they share similar architecture.

- Extreme Power Efficiency: MCUs draw a microscopic fraction of the power an SBC needs. If you are building a remote device that needs to run on a battery or solar power for months or years, an MCU is the only viable option. If you use SBC, you need a constant supply of power.

- Lower Cost and Simplicity: If a project only needs to perform one basic hardware task (like reading a temperature and turning on a fan), an MCU provides a much cheaper and simpler solution.

### Why use a Single-Board Computer over a Microcontroller?

- Heavy Computational Power: SBCs have processors capable of handling complex workloads that would instantly crash an MCU, such as running local databases, processing live video feeds, or executing machine learning/AI algorithms.

- Full Operating System Environment: Having a full OS allows for true preemptive multitasking. It also allows developers to write code in high-level languages (like Python, Java, or C++) and utilize massive software libraries, complex networking stacks, and standard file management systems.

- Native Multimedia Interfaces: SBCs come equipped with standard consumer ports like HDMI for displays, multiple USB ports for external peripherals, and built-in audio output, making them essential for projects that require a graphical user interface (GUI).