### Sensor: HC-SR04 Ultrasonic Distance Sensor

- What it does: It measures the physical distance between the sensor and an object by emitting an ultrasonic sound wave and calculating how long it takes for the echo to bounce back. Like a submarine.

- The electronics/hardware used inside: It contains two piezoelectric transducers. One acts as a speaker (the transmitter) that generates the ultrasonic sound pulse. The other acts as a microphone (the receiver) that listens for the echo. It also houses a small internal oscillator and a control circuit board to manage the signal timing.

- Analog or digital: Digital. It relies on a digital trigger pulse from the dev kit to start the measurement, and it returns a digital "echo" pulse. The length of time that echo pin stays "HIGH" determines the distance.

- Units and range of measurements: The distance is mathematically calculated and usually output in centimeters (cm) or inches. The effective measuring range is from 2 cm to 400 cm (with a general accuracy of about 3 mm).

### Actuator: SG90 Micro Servo Motor

- What it does: Unlike a standard motor that just spins continuously, a servo motor rotates to a highly specific, targeted angle and actively holds that physical position. They are used for precise mechanical movements, like steering a physical wheel or moving a robotic arm.

- The electronics/hardware used inside: It consists of a tiny, high-speed DC motor connected to a physical gear train to reduce speed and increase torque. Crucially, the output shaft is connected to an internal potentiometer (a variable resistor) that acts as a physical feedback loop, telling the internal control circuit board exactly what angle the motor is currently facing.

- Analog or digital: Digital. It is driven by a digital PWM (Pulse Width Modulation) signal from the dev kit. The dev kit sends a digital pulse of varying widths, and the internal circuit translates that width into a physical position.

- Units and range of inputs: The input command is measured in the length of the pulse width (usually between 1 and 2 milliseconds). The physical output is measured in degrees (°). The standard rotational range for the SG90 is 0° to 180°.